## DFMEA

## Design Failure Mode and Effects Analysis

# Product Functions and Failure Modes

## Appearance - Looks Good; Clean; Shiny; No defects...

Not shiny; Dirty; has Scratches;  Dents or Broken parts,

Not meeting Customer Expectations...

## Performance - Resistant to damage; Starts and runs ...

Delamination, Chipping, Cracking; Humidity Failures; Poor etch; Poor chipping; Poor adhesion; Poor scratch, mar and  solvent resistance ...

## Processability - Easy to use &  Easy to make...

Will not start; Hard to start; Hard to polish; Hard to repair...

## Environmental - Easy disposal; Low odor; Low VOC...

Hard to dispose; Smells bad / high odor; Harmful to the environment

# Design Matrix

## Material or Parts

- Selection of Materials, Parts and Quantity Levels

- Creates the Boundaries for Customer change notification

- Resin A - Failure under 47% or over 58% by Weight

## Material or Parts Characteristics

- Establish Material and Part Specifications

- Identify significant properties which have a major impact on the final product performance

- Resin A Viscosity - Failure under Z2 & over z4 Tube

## Product Characteristics

- Select tests and specifications which determine the ultimate performance of the product – Define the special characteristics

- Gel Time - Failure under 45 sec. to over 120 sec.

## Process Characteristics

- Significant manufacturing constraints

- Zone temp. - Failure under 80°C to over 150°C

## Conditions for Use

- Constraints on the proper use of the product

Film Build - Failure under 2.0 mils.